RTL DESIGN DOCUMENT

APB-Based I²C Master Controller

Register Map — Bit-Field Definitions, Reset Values & Descriptions

| Document Number | RTL-RM-I2C-APB-001 |
| --- | --- |
| Version | 1.0 |
| Date | 2026-03-11 |
| Subject | Full Register Bit-Field Reference |
| Number of Registers | 6 × 8-bit (word-aligned on 32-bit APB) |
| Base Address | 0x00000000 |
| Address Stride | 0x04 (4 bytes between registers) |

# Table of Contents

# 1. Register Map Overview

The APB-Based I²C Master Controller contains six 8-bit registers mapped into a 32-bit APB address space. Each register occupies one 32-bit word (4-byte aligned). When reading, PRDATA[31:8] always returns 0x000000; only PRDATA[7:0] carries register data. When writing, only PWDATA[7:0] is stored; PWDATA[31:8] is ignored.

| PADDR | PADDR[4:2] | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | 3'b000 | CTRL | R/W | 0x00 | Control register — enables and commands the I²C core |
| 0x04 | 3'b001 | STATUS | RO | 0x00 | Status flags — written by FSM, read by CPU |
| 0x08 | 3'b010 | TXDATA | R/W | 0x00 | Transmit data — byte to send on I²C SDA line |
| 0x0C | 3'b011 | RXDATA | RO | 0x00 | Receive data — byte captured from I²C SDA line |
| 0x10 | 3'b100 | PRESCALE_L | R/W | 0x00 | SCL clock divisor — lower byte [7:0] |
| 0x14 | 3'b101 | PRESCALE_H | R/W | 0x00 | SCL clock divisor — upper byte [15:8] |

# 2. CTRL — Control Register

## Address: 0x00   |   Access: R/W   |   Reset Value: 8'b0000_0001   (0x01)

Register Bit Layout  (Bit 7 = MSB,  Bit 0 = LSB):

| RESERVED | SPEED | IRQ_EN | ACK_EN | RD_WRN | STOP | START | EN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |

Reset Value = 8'b0000_0001   (EN=1 default, ACK_EN=1 default)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| RESERVED [Bit 7] | — | 0 | Reserved bit. Always reads as 0. Writing has no effect. Do not use. |
| SPEED [Bit 6] | R/W | 0 | SCL Speed Select. 0 = Standard Mode — 100 kHz SCL frequency (PRESCALE=0x0063 @ 50MHz PCLK) 1 = Fast Mode — 400 kHz SCL frequency (PRESCALE=0x0018 @ 50MHz PCLK) This bit does not change the SCL frequency directly — it only serves as a mode indicator. The actual frequency is determined by PRESCALE_L and PRESCALE_H registers. |
| IRQ_EN [Bit 5] | R/W | 0 | Interrupt Request Enable. 0 = Interrupt output (irq) disabled — irq pin remains LOW always 1 = Interrupt output enabled — irq asserts HIGH whenever STATUS.IF=1 Used when the CPU uses interrupt-driven I²C instead of polling STATUS.IF. |
| ACK_EN [Bit 4] | R/W | 1 | Master Acknowledge Enable (Read Mode only). 1 = Master sends ACK (SDA=0) after each received byte — requests more bytes from slave 0 = Master sends NACK (SDA=1) — signals slave to stop transmitting Only meaningful when RD_WRN=1 (read transaction). In write mode, ACK_EN is ignored. |
| RD_WRN [Bit 3] | R/W | 0 | Read / Write Direction Select. 0 = Write transaction — master transmits data to slave 1 = Read transaction — master receives data from slave This bit must be set BEFORE asserting START. It determines the R/W bit (LSB) appended to the 7-bit slave address in TXDATA. For write to slave 0x50: TXDATA=0xA0 (0x50<<1¦0), RD_WRN=0 For read from slave 0x50: TXDATA=0xA1 (0x50<<1¦1), RD_WRN=1 |
| STOP [Bit 2] | R/W | 0 | Generate STOP Condition. 0 = No STOP action 1 = FSM generates I²C STOP condition after current byte completes This bit auto-clears after the FSM processes it. Do not rely on reading it back as 1. Write STOP=1 together with EN=1. FSM will finish the current ACK phase then generate: SCL=HIGH, SDA=LOW→HIGH (STOP condition). |
| START [Bit 1] | R/W | 0 | Generate START Condition. 0 = No START action 1 = FSM initiates I²C transaction — generates START condition then begins address transmission This bit auto-clears after the FSM processes it. Write START=1 only when EN=1 and TXDATA already contains the address byte. If START=1 is written while a transaction is in progress (BUSY=1), behaviour is undefined. |
| EN [Bit 0] | R/W | 1 | I²C Core Enable. 0 = I²C core disabled — SCL and SDA are released (tri-stated). FSM stays in IDLE. 1 = I²C core enabled — SCL and SDA drivers are active. FSM can accept START commands. Must be 1 before any transaction. Reset value is 1 so the core is ready after reset. If EN is cleared while BUSY=1, the current transaction aborts immediately. |

# 3. STATUS — Status Register

## Address: 0x04   |   Access: Read-Only   |   Reset Value: 8'b0000_0000   (0x00)

Note: STATUS is written exclusively by the i2c_master_fsm through the status_wen/status_wdata ports.
CPU writes to this address are silently ignored.

Register Bit Layout:

| — | — | — | IF | AL | RXACK | TIP | BUSY |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset Value = 8'b0000_0000   (all status flags clear after reset)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| [Bits 7:5] RESERVED | — | 0 | Reserved. Always read as 0. These bits are not implemented. |
| IF [Bit 4] | RO | 0 | Interrupt Flag — Transfer Complete. 0 = No event since last clear 1 = A byte transfer has completed (address phase ACKed, or data byte ACKed/NACKed, or STOP generated) Set by FSM at the end of: ADDR_ACK state, DATA_WR_ACK state, DATA_RD_ACK state, and STOP state. CPU clears IF by writing 1 to this bit position in CTRL (it is not self-clearing). If IRQ_EN=1, the irq output pin mirrors this flag. |
| AL [Bit 3] | RO | 0 | Arbitration Lost. 0 = No arbitration loss 1 = Bus arbitration was lost (another master won control) In this single-master design, AL is always 0. It is included for compatibility with multi-master I²C implementations. |
| RXACK [Bit 2] | RO | 0 | Received Acknowledge from Slave. 0 = ACK received — slave acknowledged (SDA was LOW during 9th clock) 1 = NACK received — slave did not acknowledge (SDA remained HIGH) Updated by FSM during ADDR_ACK and DATA_WR_ACK states after sampling sda_in. CPU should check RXACK after each STATUS.IF=1. If RXACK=1, abort and check slave. |
| TIP [Bit 1] | RO | 0 | Transfer In Progress. 0 = No active bit shifting on the I²C bus 1 = FSM is currently shifting bits — either transmitting or receiving a byte Set HIGH at the start of ADDR state and DATA_WR / DATA_RD states. Cleared LOW when the 8th bit completes and FSM moves to ACK state. CPU can use TIP=0 as an alternative way to detect byte completion. |
| BUSY [Bit 0] | RO | 0 | I²C Bus Busy. 0 = Bus is free — no transaction in progress 1 = Bus is active — between a START and STOP condition Set HIGH by FSM when it generates the START condition. Cleared LOW by FSM only when the STOP condition completes. CPU must check BUSY=0 before starting a new transaction to avoid bus collisions. |

# 4. TXDATA — Transmit Data Register

## Address: 0x08   |   Access: R/W   |   Reset Value: 8'b0000_0000   (0x00)

Register Bit Layout:

| TD7 | TD6 | TD5 | TD4 | TD3 | TD2 | TD1 | TD0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset Value = 8'b0000_0000   (no transmit data loaded)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| TD[7:0] [Bits 7:0] | R/W | 0x00 | Transmit Data Bits 7 through 0. TD7 is the MSB — this is the first bit driven onto SDA. TD0 is the LSB — this is the last bit driven onto SDA. I²C protocol is MSB-first: TD7 is transmitted first. ADDRESS PHASE: CPU writes {slave_addr[6:0], RD_WRN} into TXDATA before asserting START. — Write to slave 0x50: TXDATA = 0xA0 (1010_0000) — Read from slave 0x50: TXDATA = 0xA1 (1010_0001) DATA PHASE: CPU writes raw data byte into TXDATA while slave is ACKing address. — Send byte 0xA5: TXDATA = 0xA5 (1010_0101) The FSM latches TXDATA into shift_reg by asserting load_en=1 at the start of each byte phase. CPU must ensure TXDATA is written BEFORE the FSM asserts load_en (i.e., before STATUS.IF is cleared for the next byte). |

### TXDATA Encoding Reference

| Phase | Slave Addr | TXDATA Value | Binary |
| --- | --- | --- | --- |
| Address (Write) | 0x50 | 0xA0 | 1010_0000 |
| Address (Read) | 0x50 | 0xA1 | 1010_0001 |
| Address (Write) | 0x48 | 0x90 | 1001_0000 |
| Address (Read) | 0x48 | 0x91 | 1001_0001 |
| Data Byte | Any | 0xA5 | 1010_0101 |
| Data Byte | Any | 0xFF | 1111_1111 |

# 5. RXDATA — Receive Data Register

## Address: 0x0C   |   Access: Read-Only   |   Reset Value: 8'b0000_0000   (0x00)

Note: RXDATA is written exclusively by shift_reg through the rxdata_wen/rxdata_wdata ports when 8 RX bits are complete.
CPU writes to this address are silently ignored.

Register Bit Layout:

| RD7 | RD6 | RD5 | RD4 | RD3 | RD2 | RD1 | RD0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset Value = 8'b0000_0000   (no received data)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| RD[7:0] [Bits 7:0] | RO | 0x00 | Received Data Bits 7 through 0. RD7 is the MSB — the first bit received from SDA. RD0 is the LSB — the last bit received from SDA. I²C protocol is MSB-first: RD7 arrives first and is stored in the MSB position. HOW IT IS WRITTEN: 1. FSM sets tx_mode=0 — shift_reg enters receive mode 2. For each of 8 shift_en pulses: sreg = {sreg[6:0], sda_in} 3. After 8 bits: shift_reg asserts rxdata_wen=1 and rxdata_wdata=sreg[7:0] 4. reg_file latches rxdata_wdata into RXDATA on this clock edge 5. STATUS.IF is set to 1 HOW CPU READS IT: 1. CPU polls STATUS until IF=1 2. CPU reads PADDR=0x0C — PRDATA[7:0] = RXDATA contents 3. CPU processes the received byte CPU must read RXDATA before the next byte arrives or it will be overwritten. |

# 6. PRESCALE_L — Clock Prescaler Low Byte

## Address: 0x10   |   Access: R/W   |   Reset Value: 8'b0000_0000   (0x00)

Register Bit Layout:

| P7 | P6 | P5 | P4 | P3 | P2 | P1 | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset Value = 8'b0000_0000   (prescaler not configured — must be set before EN=1)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| P[7:0] [Bits 7:0] | R/W | 0x00 | Prescaler Lower Byte — bits [7:0] of the 16-bit SCL clock divisor. Combined with PRESCALE_H to form: prescale[15:0] = {PRESCALE_H[7:0], PRESCALE_L[7:0]} FREQUENCY FORMULA: f_SCL = f_PCLK ÷ (5 × (PRESCALE + 1)) For 100 kHz SCL at 50 MHz PCLK: PRESCALE = (50,000,000 ÷ (5 × 100,000)) − 1 = 99 = 0x63 Write PRESCALE_L = 0x63, PRESCALE_H = 0x00 For 400 kHz SCL at 50 MHz PCLK: PRESCALE = (50,000,000 ÷ (5 × 400,000)) − 1 = 24 = 0x18 Write PRESCALE_L = 0x18, PRESCALE_H = 0x00 The clk_div block continuously reads prescale[15:0] and reloads its down-counter on each expiry. Must be written before setting CTRL.EN=1. |

# 7. PRESCALE_H — Clock Prescaler High Byte

## Address: 0x14   |   Access: R/W   |   Reset Value: 8'b0000_0000   (0x00)

Register Bit Layout:

| P15 | P14 | P13 | P12 | P11 | P10 | P9 | P8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset Value = 8'b0000_0000   (upper byte of prescaler — typically 0x00 for most configurations)

### Bit-Field Descriptions

| Bit Name | Access | Reset | Description |
| --- | --- | --- | --- |
| P[15:8] [Bits 7:0] | R/W | 0x00 | Prescaler Upper Byte — bits [15:8] of the 16-bit SCL clock divisor. Combined with PRESCALE_L: prescale[15:0] = {PRESCALE_H[7:0], PRESCALE_L[7:0]} For most practical configurations at 50 MHz PCLK, PRESCALE_H = 0x00 because the required prescaler values (99 for 100kHz, 24 for 400kHz) fit within 8 bits. PRESCALE_H is needed for: — Very slow SCL rates below 1 kHz — Very high PCLK frequencies (e.g. 200 MHz or above) where prescaler > 255 Example: 100kHz at 200MHz PCLK: PRESCALE = (200,000,000 ÷ (5 × 100,000)) − 1 = 399 = 0x018F PRESCALE_L = 0x8F, PRESCALE_H = 0x01 |

### Prescaler Quick Reference Table

| f_PCLK | f_SCL | PRESCALE_H | PRESCALE_L | Combined PRESCALE[15:0] |
| --- | --- | --- | --- | --- |
| 50 MHz | 100 kHz | 0x00 | 0x63 | 0x0063 (99 decimal) |
| 50 MHz | 400 kHz | 0x00 | 0x18 | 0x0018 (24 decimal) |
| 100 MHz | 100 kHz | 0x00 | 0xC7 | 0x00C7 (199 decimal) |
| 100 MHz | 400 kHz | 0x00 | 0x31 | 0x0031 (49 decimal) |
| 200 MHz | 100 kHz | 0x01 | 0x8F | 0x018F (399 decimal) |
| 200 MHz | 400 kHz | 0x00 | 0x63 | 0x0063 (99 decimal) |

# 8. Complete Register Bit Summary

The table below shows all 6 registers side by side — every bit, its access type, reset value, and one-line function description.

| Register | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CTRL (0x00) | RSVD | SPEED | IRQ_EN | ACK_EN | RD_WRN | STOP | START | EN |
| STATUS (0x04) | — | — | — | IF | AL | RXACK | TIP | BUSY |
| TXDATA (0x08) | TD7 | TD6 | TD5 | TD4 | TD3 | TD2 | TD1 | TD0 |
| RXDATA (0x0C) | RD7 | RD6 | RD5 | RD4 | RD3 | RD2 | RD1 | RD0 |
| PRESCALE_L (0x10) | P7 | P6 | P5 | P4 | P3 | P2 | P1 | P0 |
| PRESCALE_H (0x14) | P15 | P14 | P13 | P12 | P11 | P10 | P9 | P8 |

## 8.1  Reset Values Summary

| Register | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CTRL (0x01) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| STATUS (0x00) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TXDATA (0x00) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| RXDATA (0x00) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PRESCALE_L (0x00) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PRESCALE_H (0x00) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

# 9. Register Access Error Conditions

The apb_slave_if asserts PSLVERR=1 for any of the following illegal accesses. PREADY remains HIGH even on error — the transaction still completes in 2 cycles.

| Error Condition | Example PADDR | Explanation |
| --- | --- | --- |
| PADDR > 0x14 (out of range) | 0x00000018 | No register exists above 0x14. Upper address bits non-zero trigger PSLVERR. |
| Unaligned: PADDR[1:0] = 2'b01 | 0x00000001 | Byte address. APB requires word alignment (multiples of 4 only). |
| Unaligned: PADDR[1:0] = 2'b10 | 0x00000002 | Half-word address. Not permitted on 32-bit APB. |
| Unaligned: PADDR[1:0] = 2'b11 | 0x00000003 | Byte address +3. Not permitted. |
| CPU write to STATUS (0x04) | 0x00000004 + PWRITE=1 | Write is accepted (no PSLVERR) but silently ignored. STATUS is read-only. |
| CPU write to RXDATA (0x0C) | 0x0000000C + PWRITE=1 | Write accepted but silently ignored. RXDATA is read-only. |

# Appendix — Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2026-03-11 | Initial release — all 6 registers with bit-field diagrams, reset values, and descriptions |